Experiment Number: 2

MATLAB Simulation of Cellular Coverage, Cell Splitting and Handoff in a Multi-Cell Network

Aim

To model a simplified hexagonal cellular network and study how cell radius affects coverage, estimated user capacity and handoff as a mobile crosses a cell boundary.

Objectives

By the end of the experiment, students should be able to:

- Model adjacent hexagonal cell coverage and identify cell centers and boundaries.

- 2. Estimate the number of cells required to cover a fixed service area for different cell radii.

3. Compare estimated users served under an explicit per-cell capacity assumption.

4. Simulate a mobile moving across a cell boundary and identify the serving-cell transition.

Requirements

• Computer with MATLAB R2019b or later.

• No RF hardware is required; this is a geometric and analytical simulation.

Theoretical Background

A regular hexagonal cell is an idealized coverage region. For cell radius R, measured from the center to a vertex, its area is A_cell = (3√3/2)R². A first-order estimate for a service area A is N_cells = ceil(A/A_cell). This area ratio ignores boundary overlap, roads, terrain, irregular coverage and the fact that real cell layouts extend beyond the service boundary.

For a simple capacity comparison, assume each cell can serve C users at the same time. Estimated network capacity is N_cells × C. This is a planning illustration, not a prediction of a real network; capacity also depends on spectrum, scheduling, traffic, interference, backhaul and radio conditions.

For handoff, the serving cell is approximated as the cell center nearest to the mobile. A handoff is indicated when the nearest-center identity changes. Real systems use measured radio quality, hysteresis, time-to-trigger, load and network policy, so this geometric rule is only a teaching model.

Parameters and Assumptions

Record any values changed. The capacity comparison holds the per-cell capacity constant to isolate the effect of cell count; it does not imply that smaller real cells always have exactly this capacity.

Procedure

1. Run the MATLAB script below and record the cell area, estimated cell count and total illustrative capacity for each radius.

2. Inspect the hexagonal layout and confirm that adjacent cells share edges without gaps in the idealized tiling.

3. Inspect the mobile-path plot and note the simulated handoff point.

4. Change the service area, radii or users-per-cell assumption one at a time; rerun and compare results.

5. Explain why the geometric cell count and nearest-center handoff differ from an operational cellular network.

Program code

% Experiment 2: Simplified Hexagonal Coverage and Handoff

clear;

clc;

close all;

%% 1. Baseline assumptions

serviceAreaKm2 = 5;

radiiKm = [1.0 0.5];

usersPerCell = 30;

% Area of a regular hexagonal cell

hexArea = @(R) (3 * sqrt(3) / 2) .* R.^2;

cellAreaKm2 = hexArea(radiiKm);

cellCount = ceil(serviceAreaKm2 ./ cellAreaKm2);

estimatedUsers = cellCount .* usersPerCell;

% Display capacity table

capacityTable = table( ...

radiiKm(:), ...

cellAreaKm2(:), ...

cellCount(:), ...

repmat(usersPerCell, numel(radiiKm), 1), ...

estimatedUsers(:), ...

'VariableNames', {'Radius_km', 'CellArea_km2', ...

'EstimatedCells', 'UsersPerCell', 'IllustrativeUsers'});

disp('Cell Coverage and Capacity Table:');

disp(capacityTable);

%% 2. Plot a seven-cell hexagonal cluster

% One central cell and six surrounding cells

R = radiiKm(1);

centers = [

0,             0;

1.5*R,         sqrt(3)*R/2;

1.5*R,        -sqrt(3)*R/2;

0,             sqrt(3)*R;

0,            -sqrt(3)*R;

-1.5*R,         sqrt(3)*R/2;

-1.5*R,        -sqrt(3)*R/2

];

theta = (0:6) * pi/3;

figure;

hold on;

axis equal;

grid on;

for k = 1:size(centers, 1)

xHex = centers(k,1) + R*cos(theta);

yHex = centers(k,2) + R*sin(theta);

plot(xHex, yHex, 'k-', 'LineWidth', 1.2);

text(centers(k,1), centers(k,2), ...

sprintf('Cell %d', k), ...

'HorizontalAlignment', 'center');

end

xlabel('x (km)');

ylabel('y (km)');

title('Idealized Seven-Cell Hexagonal Cluster');

hold off;

%% 3. Mobile movement and handoff calculation

% Mobile moves from cell 1 toward cell 2

mobileX = linspace(-0.2*R, 1.7*R, 300);

mobileY = zeros(size(mobileX));

% Distance from mobile to the two cell centers

d1 = hypot(mobileX - centers(1,1), ...

mobileY - centers(1,2));

d2 = hypot(mobileX - centers(2,1), ...

mobileY - centers(2,2));

% Nearest-center serving-cell selection

servingCell = ones(size(mobileX));

servingCell(d2 < d1) = 2;

% Find the first handoff point

handoffIndex = find(diff(servingCell) ~= 0, 1, 'first');

%% 4. Plot distances and handoff location

figure;

hold on;

grid on;

plot(mobileX, d1, 'b-', 'LineWidth', 1.5);

plot(mobileX, d2, 'm-', 'LineWidth', 1.5);

if ~isempty(handoffIndex)

xHandoff = mobileX(handoffIndex + 1);

xline(xHandoff, '--r', 'Handoff decision', ...

'LineWidth', 1.2);

end

xlabel('Mobile x-position (km)');

ylabel('Distance to cell center (km)');

legend('Cell 1', 'Cell 2', 'Location', 'best');

title('Nearest-Center Handoff Approximation');

hold off;

%% 5. Plot serving-cell ID along the mobile path

figure;

stairs(mobileX, servingCell, 'LineWidth', 1.5);

grid on;

xlabel('Mobile x-position (km)');

ylabel('Serving-cell ID');

yticks([1 2]);

ylim([0.5 2.5]);

title('Serving Cell Along the Mobile Path');

%% 6. Display handoff result

if ~isempty(handoffIndex)

fprintf('Geometric handoff at x = %.3f km\n', ...

mobileX(handoffIndex + 1));

else

fprintf('No handoff detected along the mobile path.\n');

end

output